Text 3

DESCALING INSTRUCTIONS - IN HARD WATER AREAS REGULAR DESCALING IS ESSENTIAL TO KEEP YOUR KETTLE IN GOOD WORKING ORDER. - If your kettle switches off before it has boiled it indicates your kettle needs descaling. - Descale your kettle using - either a proprietary kettle descaler suitable for PLASTIC kettles. Follow manufacturer's instructions, - or white vinegar - Fill the kettle with ½ l of 8° white vinegar. - Leave to soak for 1 hour without boiling. - Empty your kettle and rinse it two or three times with clear water. - or citric acid - Boil 0.5 l of water. - Add 25 g of citric acid. - Leave to soak for 15 minutes. - Discard this mixture. - Rinse the kettle with clear water two or three times. - Repeat process if needed. - Descaling agents for metal kettles, coffee makers or steam irons are unsuitable and will damage your Tefal kettle. Never use metal abrasives to clean the interior. - When using powder descaler or citric acid, always add water. Never use in an empty kettle. - Always follow the directions for use on the descaler carefully. Rinse the inside well after using a descaler.